Supplementary Material: Failure-Aware Smartphone Inertial Dead Reckoning Under GNSS Outages

Article Title: Failure-Aware Smartphone Inertial Dead Reckoning Under GNSS Outages: An Empirical Study of Learned Velocity, Sensor Fusion, Map Constraints, and Distribution Shift

Journal: The Journal of Navigation (Cambridge University Press / Royal Institute of Navigation)

Author: Rahul Kumar

Affiliation: Integrated M.Tech. (Materials Engineering), School of Engineering Sciences & Technology, University of Hyderabad, Hyderabad, India

Corresponding Email: 24f2001869@ds.study.iitm.ac.in

Overview

This supplementary document provides extended tabular data, detailed per-trip forensic metrics, hardware profiling logs, and mathematical derivations supporting the main article. All evaluations utilize the public IO-VNBD benchmark dataset (Onyekpe et al., 2021) and on-device Android mobile execution on a Google Pixel 7a smartphone.

Supplementary Table S1: Per-Trip Performance Breakdown Under 60-Second GNSS Blackouts

The main manuscript presents aggregate and category-level metrics across the 13 usable held-out test routes. Table S1 provides the comprehensive route-by-route results for both unassisted strapdown inertial dead reckoning and the proposed NavSense adaptive fusion engine.

Table S1. Detailed route-by-route evaluation across all 13 usable held-out test trips under 60-second satellite outages.

| Trip ID | Route Environment | Reference Distance (m) | Pure Strapdown Drift (m) | Pure Strapdown Drift (%) | NavSense Adaptive Drift (m) | NavSense Adaptive Drift (%) | SIH Benchmark Status (<10%) | Observed Dynamic Characteristics |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vta21 | Suburban Town | 786.7 | 406.8 | 51.45% | 61.5 | 7.88% | MET (PASS) | Steady suburban cruising along arterial corridor; bounded drift |
| Vta22 | Suburban Town | 641.3 | 176.7 | 27.72% | 107.7 | 16.84% | Unmet | Frequent stop-and-go queue; velocity dampens with traffic |
| Vta23 | Suburban Town | 570.1 | 352.9 | 62.40% | 156.8 | 26.16% | Unmet | Sharp 90-degree intersection turn during blackout window |
| Vta24 | Suburban Town | 384.8 | 293.6 | 73.42% | 57.0 | 14.65% | Unmet | Commercial zone corridor; low-speed transient manoeuvres |
| Vta26 | Suburban Town | 258.7 | 292.8 | 574.65% | 66.3 | 47.31% | Unmet | Short transit crawl; large percentage due to short travel distance |
| Vta27 | Suburban Town | 818.9 | 306.5 | 37.18% | 95.9 | 12.68% | Unmet | Signalised arterial road with moderate speed transitions |
| Vta28 | Suburban Town | 565.3 | 281.8 | 64.92% | 120.6 | 25.30% | Unmet | Roundabout entry and deceleration into residential zone |
| Vw12 | Mountain Route | 1500.2 | 650.3 | 43.40% | 45.3 | 3.01% | MET (PASS) | Continuous mountain descent; highly dynamic centripetal cues |
| Vw14a | Mountain Route | 1511.1 | 312.6 | 21.06% | 130.4 | 8.54% | MET (PASS) | Rural mountain corridor with sweeping curves and steady throttle |
| Vw14b | Mountain Route | 1264.5 | 432.8 | 42.48% | 121.0 | 10.19% | Unmet | Consecutive hairpins; centripetal acceleration transitions |
| Vw16a | Mountain Route | 899.9 | 388.5 | 51.76% | 136.1 | 16.88% | Unmet | High curvature hill climb with frequent throttle modulations |
| V-Vfa02 | Motorway | 1460.3 | 313.9 | 25.44% | 157.2 | 11.69% | Unmet | Unaccelerated high-speed cruising; negative TCN regression bias |
| Vw15 | Stationary Control | 1.3 | 5.6 | 450.48% | 0.82 | 63.83% | Unmet | Zero-velocity control trip; standstill clamped by causal ZVD |
| Mean / Total | All 13 Routes | 820.2 | 324.21 | 89.66% | 96.65 | 16.76% | 3 / 13 (23.08%) | Complete Held-Out 60-s Evaluation Set |

Note: In accordance with the experimental protocol, trips with total durations shorter than 70.0 seconds are excluded from the 60-second evaluation set because they cannot accommodate the 10-second pre-outage calibration window plus the 60-second blackout duration. Macro-average normalized drift weights each dynamic trip equally, excluding the stationary control trip Vw15 from the percentage mean per Equation (13) to avoid distortion from the near-zero travel-distance denominator. Normalised drift percentages are calculated using the reference distance travelled during the corresponding 60-s blackout window, S_i(T), as defined in Eq. (13). The 'Reference Distance' column reports the route/reference distance associated with the evaluation record and is not necessarily identical to the blackout-window travel distance.

Supplementary Table S2: Spectral Vibration Speed Audit Across 64 Vehicle Trips

To evaluate the hypothesis that vehicle road and engine vibration captured by consumer smartphone IMUs provides a reliable proxy for vehicle forward speed, Table S2 summarizes the spectral census across all 64 passenger-car trips in the IO-VNBD dataset (>450,000 one-second FFT windows).

Table S2. Cross-trip spectral vibration census across four standardized road categories.

| Road Environment | Trips Analyzed (N) | Total 1-s Windows | Dominant Spectral Peak (Hz) | Pearson Correlation (r) with Speed | Spearman Correlation (ρ) with Speed | Physical Interpretation |
| --- | --- | --- | --- | --- | --- | --- |
| Suburban Towns (Vta) | 30 | 218450 | 2.24 ± 0.18 Hz | -0.028 | -0.024 | Consistent with low-frequency vehicle-body/chassis dynamics |
| Dense Urban (Vtb) | 12 | 40890 | 2.31 ± 0.22 Hz | -0.041 | -0.035 | Consistent with road-surface excitation and low-frequency vehicle dynamics |
| Mountain Routes (Vw) | 20 | 170920 | 2.45 ± 0.15 Hz | -0.019 | -0.016 | Consistent with chassis/body dynamics under curved-road motion |
| Motorways (V-Vfa) | 2 | 39180 | 2.86 ± 0.12 Hz | -0.038 | -0.031 | Persistent spectral peak across speed bands; physical mechanism not directly identified |
| Complete Corpus | 64 | >450,000 | 2.2–2.5 Hz | -0.032 | -0.028 | No reliable speed signal identified at 10 Hz feature rate |

Methodological Note: Power spectral density was computed causally using Welch periodograms across 1.0-second sliding windows (10 samples, zero-padded to 64 points) on tri-axial specific forces and angular velocities.

Supplementary Table S3: On-Device Android Execution Profiling and Numerical Parity

Table S3 details the execution latency and cross-platform numerical parity between the mobile Android deployment (Java/EJML/ONNX Runtime) on a Google Pixel 7a and the 64-bit Python reference engine.

Table S3. Mobile execution benchmarks and cross-platform numerical parity on Google Pixel 7a.

| Pipeline Subsystem | Implementation Language | Runtime Framework | Mean Execution Latency | Peak Latency (99th %tile) | Maximum Discrepancy vs. Python 64-bit Engine |
| --- | --- | --- | --- | --- | --- |
| IMU Ingestion & Decimation | Kotlin | Android SensorManager HAL | 0.12 ms | 0.35 ms | Identical (Zero difference) |
| Coordinate Leveling & PCA | Java | Native Linear Algebra | 0.24 ms | 0.58 ms | Leveling Angle: <0.005° |
| TCN Forward Inference | C++ / Java Bindings | ONNX Runtime Mobile v1.16 | 5.82 ms | 7.84 ms | Speed: <0.001 m/s |
| 15-State ESKF Propagation | Java | Efficient Java Matrix Library (EJML) | 0.94 ms | 1.38 ms | Position: 0.24177 m; Heading: 0.01829° |
| Decoupled Velocity Damping | Java | Custom EJML Matrix Slice | 0.18 ms | 0.32 ms | Velocity: 0.02117 m/s |
| Open-Loop Vector Mapping | Kotlin | OSMDroid Offline Engine | 1.85 ms | 3.42 ms | Render: <1 frame |
| Total Navigation Epoch Loop | Mixed | End-to-End Android Pipeline | 9.15 ms | 13.89 ms | Within the 100-ms epoch budget |

Supplementary Section S1: Mathematical Derivation of Causal Receptive Field Reach

The causal Temporal Convolutional Network (TCN) operates over a sliding temporal buffer of W=100 samples (10.0 s at 10 Hz). To ensure that predictions depend strictly on past motion dynamics with zero dependency on future epochs, causal convolutions are enforced via asymmetric left-padding.

For a 1D convolutional network with L dilated residual blocks, kernel size k, and dilation factor dl at layer l, the mathematical receptive field reach R (expressed in sample steps) is given by:

R=1+l=0L−1(k−1)·dl

In the TCN architecture, the parameters are:

* Number of dilated residual layers: L=4

* Convolutional kernel size: k=3

* Dilation schedule: dl∈{1,2,4,8} for l∈{0,1,2,3}

Each residual block contains two sequential dilated convolutional layers with identical dilation dl. Substituting these values:

Rblock,l=2·(k−1)·dl=2·(3−1)·dl=4dl

Summing across all four dilated blocks:

R=1+l=034dl=1+4·(1+2+4+8)=1+4·15=61 samples

At the nominal 10.0 Hz sampling rate, 61 samples correspond to an effective causal look-back interval of:

τcausal=(61−1)×0.10 s=6.0 seconds

representing input dependency on the causal interval t−60,t.